| Alcohol-based prep solution dried for >5 minutes. No pooling observed. Yes No NA |  |  |  |
| --- | --- | --- | --- |
| (Circle appropriate option) | Y | N |  |
| * Surgical site or incision above the xiphoid, or involving airway or pulmonary components | 1 | 0 |  |
| * Open oxygen source, >40% oxygen (supplemental oxygen via face mask or nasal cannula) potential airway leak, proximity of ETT, double-lumen tube | 1 | 0 |  |
| * Available ignition source: i.e. monopolar electrosurgery unit, laser, fiberoptic light source | 1 | 0 |  |
| Total score |  |  |  |
| Scoring: 3 = High risk 2= Low risk w/potential to convert to high risk 1= Low risk |  |  |  |
| High Risk Fire Protocol initiated by: Anesthesia provider Surgeon RN Scrub tech (circle one) |  |  |  |

Fire Risk Protocols:

Score 3 = High Risk

The circulating nurse, surgeon and anesthesia providers take these precautions and communicate at handoff:

Circulating nurse:

- Write “Fire Risk High” on dry erase board and fill out the Red Fire Triangle.

- Ensures appropriate draping techniques to minimize oxygen

- Suction by O2 prongs to “scavenge” O2

- Provides sterile carbon dioxide flush line with filter to surgical tech. Ensures at least 5 liters/minute of carbon dioxide flush.

- Maximizes the perimeter around the incision point.

- Confirms verbally the heat source setting.

- Assesses that enough time has been allowed for fumes of alcohol-based prep solutions to dissipate (minimum of 3 min)

- Use of saline-dampened sponges

- Basin of sterile saline and bulb syringe are available for fire suppression

- Places laser in “standby” mode when not in use. Secures laser foot pedal to prevent accidental activation

Anesthesia provider:

- Notifies the surgeon and documents if O2 concentration >40% or risk of air leak present

- Before an ignition source is activated:

  - Reduce the oxygen concentration to 40% or less if possible

  - Stop the use of nitrous oxide

  - Check for appropriate use of carbon dioxide flush system.

Surgical Tech:

- Water or saline available for the surgical field.

- Wet sponges

- Suction always available on field

- ESU in holster when not in use; light source turned off when not in use

- Positions sterile carbon dioxide flush line with filter in surgical field. Ensures at least 5 liters/minute of carbon dioxide flush.

Surgeon:

- Before an ignition source is activated:

  - Wet sponges used as barrier between ESU and oxygen source

  - Announces the initial intent to use an ignition source

  - Verifies that the anesthesia provider has reduced the O2 concentration to the minimum acceptable level for 1-3 min before using ignition source.

  - Confirm verbally the heat source setting – minimize ESU setting if possible

  - Positions sterile carbon dioxide flush line with filter in surgical field. Ensures at least 5 liters/minute of carbon dioxide flush.

In Case of Fire:

- Shout “Fire”

- Remove ETT (if airway fire)

- Turn off O2

- Throw saline on field							Revised 11/16/11aw